| Drill Bit Specifications: |  |  |
| --- | --- | --- |
| Cutter Size (Primary, Sec) | 5/8”,1/2” |  |
| Cutter Count (Total, Gage, Back Ream) | 42,16 |  |
| Blade Count | 6 |  |
| Gage Length (Inches) | 2.00 |  |
| Nozzle Count (Primary, Sec.) | 6 |  |
| Nozzle Series (Primary, Sec.) | 60 |  |
| JSA (Sq. Inches) | 14.67 |  |
| Max. TFA (Sq. Inches) | 1.49 |  |
| Operating Parameters: |  |  |
| Max. Hydraulic Flow Rate (GPM) | 810 |  |
| Max. Weight on Bit (1,000 Lbs.) | 60 |  |
| Make-up Torque (ft/lbs) | 17,399 |  |
| Nominal Dimensions: |  |  |
| Overall Length (Inches) | 14.30 |  |
| Make Up Length (Inches) | 10.20 |  |
| Bit Breaker | SKB-21010 |  |
| Connection | Ø4-1/2" API Reg. |  |
| Approx. Weight (Lbs.) | 105 |  |

Tailor-Made Features:

Premium grade cutters maximize ROP and Life

Large open face volume assists cuttings removal

Six nozzles optimize blade cleaning and cuttings removal

616-SR (BR) Nomenclature Explained:

| - • | - 6 | - = | - Number of Blades |
| --- | --- | --- | --- |
| - • | - 16 | - = | - Primary Cutter Size (16mm) |
| - • | - SR | - = | - Single Row of Cutters |
| - • | - BR+ | - = | - Back Reaming Cutters |