1991  8½ 419-SR6  Design Summary

Designed to run in lateral section on Rotary Steerable Systems through the Marcellus Shale in PA. Has different features than the 1989 designed to run on Steerable Motor System.

1991 features include:

- Aggressive design to penetrate relatively soft shale.

- 12° backrake in cone, 15° nose and shoulder, 25° gauge

- High exposure

- Tapered blade back

- Blade aspect ratio = .94

- Six series 60 nozzles

- All full round cutters on gauge

- Tapered gauge for changing angle when pushed

- Support knobs behind cutters in cone

- 12.5° cone angle

- Target ROP and RPM-	200Ft/Hr @ 200RPM