Name: Sterling Robinson

Date: 12/4/12

PDC Bit Design Request

Bit Type

Bit Diameter: 8.5”

Number of Blades: 6

Primary Cutter Size: 19

Single or Double Row: Single

Application Details

Which customers will be using the bit? PNR Permian

In what geographic locations will the bit be used? Permian

Expected footage drilled under each of these conditions: 10,000’

|  |  | Compressive Strength (unconfined) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Low <10 ksi | Medium 15 to 25 ksi | High >30 ksi |
| Formation | Shale | In: Out: | In: 7500 Out: 17500 | In: Out: |
|  | Sand | In: Out: | In: Out: | In: Out: |
|  | Lime | In: Out: | In: Out: | In: Out: |
|  | Other (Specify) | In: Out: | In: Out: | In: Out: |

Which of these footage intervals are problem formations? Issues can arise holding tool face or steering in the latter part of the lateral.

What primary performance improvement is most important vs currently used bits?

- ROP

- Footage or - Better dull condition

- Improved hole quality (directional control or smoother hole) Directional Control, Tracking

Drive Type (frequency of each anticipated)

- Rotary - RPM? Rotary 50-100 (80 mostly)

- Performance motor drilling vertical hole - RPM? 400-600 gpm on .288motor = 115-175rpm

- Steerable Motor

tangent, curve, lateral? lateral

max build rate?

rotary RPM?

motor RPM?

Revs/Gal? .28

GPM? 400-600

% sliding? Varies

%rotating? Varies

- Rotary Steerable System - max build rate? RPM?

PDC Bit Design Request - page 2

What WOB and RPM conditions are expected?  List by interval if there are variations. 25k-30k WOB

What range of instantaneous ROP is expected?  List by interval if there are variations. 150-200fph – FXD55, 270-300fph Q606

What are the current bits of choice? FXD55, Q606

Are spec sheets and/or sharp bit photos available? In progress.

What are the typical dull condition and failure mechanisms of these bits of choice? 0-0 – 1-1-WT

Are dull bit photos available?

Sales and Customer Preferences

Provide a number or description to specify preferences for any of these characteristics:

|  | Salesman Preference | Customer Preference |
| --- | --- | --- |
| Leached Cutters (Yes or No) | Yes | Yes |
| Bit Length | NP | NP that I know of |
| Gauge Length | 3” | ?” |
| Number of Blades to Center | 3 | 3 |
| Number of Nozzles | 6 | 6 |
| Maximum TFA | 2.0 | 2.0 |
| Junk Slot Area | NP | NP |
| Open Face Volume | NP | NP |
| Knuckles? (Yes or No) | No | No (depends on current benchmark and DD preference) |
| Any Other Features |  |  |

Is there any other characteristic needing special attention for this design?

To be discussed.