Name: Sterling Robinson

Date: 01/14/2013

PDC Bit Design Request

Bit Type (105 616-YR version 11 616-YR)

Bit Diameter: 10.625”

Number of Blades: 6

Primary Cutter Size: 16

Single or Double Row: Double

Application Details

Which customers will be using the bit? OXY Permian

In what geographic locations will the bit be used? Permian

Expected footage drilled under each of these conditions: 3000’-4000’

|  |  | Compressive Strength (unconfined) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Low <10 ksi | Medium 15 to 25 ksi | High >30 ksi |
| Formation | Shale | In: Out: | In: 1800 Out: 5500 | In: Out: |
|  | Sand | In: Out: | In: Out: | In: Out: |
|  | Lime | In: Out: | In: Out: | In: Out: |
|  | Other (Specify) | In: Out: | In: Out: | In: Out: |

Which of these footage intervals are problem formations? None

What primary performance improvement is most important vs currently used bits?

- ROP

- Footage or Depth target

- Better dull condition

- Improved hole quality (directional control or smoother hole)

Drive Type (frequency of each anticipated)

- Rotary - RPM? Rotary 60-120

- Performance motor drilling vertical hole - RPM?

- Steerable Motor

tangent, curve, lateral?

max build rate?

rotary RPM?

motor RPM?

Revs/Gal?

GPM?

% sliding?

%rotating?

- Rotary Steerable System - max build rate? RPM?

PDC Bit Design Request - page 2

What WOB and RPM conditions are expected?  List by interval if there are variations. 25k-35k

What range of instantaneous ROP is expected?  List by interval if there are variations. 150-250fph

What are the current bits of choice? U616M

Are spec sheets and/or sharp bit photos available? I’ll see.

What are the typical dull condition and failure mechanisms of these bits of choice? 1-1 CT -3-3- WT

Are dull bit photos available? I’ll see.

Sales and Customer Preferences

Provide a number or description to specify preferences for any of these characteristics:

|  | Salesman Preference | Customer Preference |
| --- | --- | --- |
| Leached Cutters (Yes or No) | Yes | Yes |
| Bit Length | NP | NP that I know of |
| Gauge Length | 3” | 3” |
| Number of Blades to Center | 3 | NP |
| Number of Nozzles | 9 | 6-9 |
| Maximum TFA | 2.0 | 2.0 |
| Junk Slot Area | NP | NP |
| Open Face Volume | NP | NP |
| Knuckles? (Yes or No) | No | Maybe |
| Any Other Features |  |  |

Is there any other characteristic needing special attention for this design?