Name: Sterling Robinson

Date: 4/9/13

PDC Bit Design Request

Bit Type

Bit Diameter: 8.75”

Number of Blades: 6

Primary Cutter Size: 19

Single or Double Row: Single

Application Details

Which customers will be using the bit? OXY South Texas

In what geographic locations will the bit be used? Deep South Texas

Expected footage drilled under each of these conditions: 500’-5500.

|  |  | Compressive Strength (unconfined) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Low <10 ksi | Medium 15 to 25 ksi | High >30 ksi |
| Formation | Shale | In: Out: | In: 2500 Out: 10,000 | In: Out: |
|  | Sand | In: Out: | In: 2500 Out: 10,000 | In: Out: |
|  | Lime | In: Out: | In: 2500 Out: 10,000 | In: Out: |
|  | Other (SALT) | In: Out: | In: 500 Out: 5500 | In: Out: |

Which of these footage intervals are problem formations? Some concerns with packing off. Should be able to blow through most but there are some transisitons.

What primary performance improvement is most important vs currently used bits?

- ROP

- Footage or Depth target Consistently making it to TD

- Better dull condition

- Improved hole quality (directional control or smoother hole)

Drive Type (frequency of each anticipated)

- Rotary - RPM? Rotary 150 – 225 depending on rig. 60 is the most common

- Performance motor drilling vertical hole - RPM? - Steerable Motor

tangent, curve, lateral?

max build rate?

rotary RPM?

motor RPM?

Revs/Gal?

GPM?

% sliding?

%rotating?

- Rotary Steerable System - max build rate? RPM?

PDC Bit Design Request - page 2

What WOB and RPM conditions are expected?  List by interval if there are variations. 15-25k WOB (lighter weights)

What range of instantaneous ROP is expected?  List by interval if there are variations. 150-250fph

What are the current bits of choice? CDI 519-619. Sometimes with back ups.

Are spec sheets and/or sharp bit photos available? See attached photos.

What are the typical dull condition and failure mechanisms of these bits of choice? 1-1 CT/WT – 2-2 CT/WT, Mostly green

Are dull bit photos available? Nope

Sales and Customer Preferences

Provide a number or description to specify preferences for any of these characteristics:

|  | Salesman Preference | Customer Preference |
| --- | --- | --- |
| Leached Cutters (Yes or No) | Yes | Yes, CDI spoke to them about leached vs non-leached |
| Bit Length | NP | NP |
| Gauge Length | 3” | 2-3” |
| Number of Blades to Center | 3 | NP |
| Number of Nozzles | 6 | 5-6 |
| Maximum TFA | 2.0 | 2.0 |
| Junk Slot Area | Very large | Very Large |
| Open Face Volume | Very Large | NP |
| Knuckles? (Yes or No) | No | No |
| Any Other Features | Might be a good place for a 6-3 design after testing a regular design. |  |

Is there any other characteristic needing special attention for this design? As much gauge protection as possible.