| Drill Bit Specifications: |  |  |
| --- | --- | --- |
| Cutter Size (Primary, Sec) | 3/4”,3/4” |  |
| Cutter Count (Total, Gage, Back Ream) | 37, 6 |  |
| Blade Count | 5 |  |
| Gage Length (Inches) | 3.00 |  |
| Nozzle Count (Primary, Sec.) | 7 |  |
| Nozzle Series (Primary, Sec.) | 60 |  |
| JSA (Sq. Inches) | 47.55 |  |
| Max. TFA (Sq. Inches) | 2.60 |  |
| Operating Parameters: |  |  |
| Max. Hydraulic Flow Rate (GPM) | 810 |  |
| Make-up Torque (ft/lbs) | 40,563 |  |
| Nominal Dimensions: |  |  |
| Overall Length (Inches) | *17.86 |  |
| Make Up Length (Inches) | *12.98 |  |
| Bit Breaker | SKB-21011 |  |
| Connection | Ø6-5/8" API Reg. |  |
| *Long tool joint available upon request (adds 6” to fishing neck, makeup and overall length, and 64 lbs. to approximate weight). Approx. Weight (Lbs.) | *195 |  |

Taylor-Made Features:

Premium grade cutters maximize ROP and Life

Nozzle configuration and large open face volume optimize blade cleaning and cuttings removal

519-SR7 Nomenclature Explained:

| - • | - 5 | - = | - Number of Blades |
| --- | --- | --- | --- |
| - • | - 19 | - = | - Primary Cutter Size (19mm) |
| - • | - SR | - = | - Single Row Cutters |
| - • | - 7 | - = | - Number of Nozzles |