| Drill Bit Specifications: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Cutter Size (Primary, Sec) |  | 5/8”, 1/2” |  |  |
| Cutter Count (Total, Gage) |  | 32, 8 |  |  |
| Blade Count |  | 5 |  |  |
| Gage Length (Inches) |  | 2.50 |  |  |
| Nozzle Count (Primary, Sec.) |  | 7 |  |  |
| Nozzle Size (Primary, Sec.) |  | 60 |  |  |
| JSA (Sq. Inches) |  | 18.40 |  |  |
| Max. TFA (Sq. Inches) |  | 2.60 |  |  |
| Operating Parameters: |  |  |  |  |
| Max. Hydraulic Flow Rate (GPM) |  |  |  | 810 |
| Make-Up Torque (FT-LBS) |  |  |  | 16,500 |
| Max. Weight on Bit (1,000 Lbs.) |  |  |  | 27 |
| Nominal Dimensions: |  |  |  |  |
| Overall Length (Inches) | 14.375 |  |  |  |
| Make Up Length (Inches) | 10.25 |  |  |  |
| Bit Breaker | SKB-21010 |  |  |  |
| Connection | Ø4-1/2" API Reg. |  |  |  |
| Approx. Weight (Lbs.) | 120 |  |  |  |

Standard Features:

- Large open face volume assists cuttings removal

- Seven nozzles optimize blade cleaning and cuttings removal

- Premium grade cutters maximize ROP and Life

516-SR7  Nomenclature Explained:

- 5  =  Number of Blades

- 16 =  Primary Cutter Size (16mm)

- SR =  Single Row

- 7  =  Number of Nozzles
Interchangeable nozzles.