2028A Ø8-1/2” 513-YR7 (BR)

Bit Body:

Ø8-1/2”

5 Blades

3 primary blades

2 secondary blades

75/65/80/75/65 Blade angles

7.5° Cone

1.5” Gage

Open Face Volume = 28.443 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.286” | 0.937” | 1.37 |
| Center of round | 1.597” | 1.810” | 0.88 |
| Gauge | 1.323” | 2.045” | 0.65 |

Primary Cutting Structure:

3 Pattern start to position 8; Forward Spiral; Non-Redundant;

Cut pattern is balanced.

Secondary blades are equally exposed;

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15°-30°

Gage		30°

Secondary Cutting Structure:

The Tilt angle is the same as the primary cutting structure;

Backup cutters are under exposed by 0.040”

Backup cutters are trailing by one blade.

Components:

06 - 22/32 Nozzle Sleeves

06 - 60 Series Nozzles

56 - 1313 Primary Cutters

5   - 1313 Back Reaming Cutters