2036A Ø17-1/2” 616-SR9

Bit Body:

Ø17-1/2”

6 Blades

3 primary blades

3 secondary blades

55/60/55/65/60/65 Blade angles

15° Cone

3.5” Gage

Open Face Volume = 313.422 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 2.538” | 1.485” | 1.709 |
| Center of round | 3.506” | 2.402” | 1.460 |
| Gauge | 2.789” | 2.962” | 0.942 |

Primary Cutting Structure:

3 Pattern start to position 10; Forward Spiral; Opposed redundant;

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15°-25°

Gage		30°

Components:

09 - 22/32 Nozzle Sleeves

09 - 60 Series Nozzles

69 - 1613 Primary Cutters

13 - 1313 Cutters