2046A Ø6-1/8” 313-YR

Bit Body:

Ø6-1/8”

3 Blades

3 primary blades

130/115/115 Blade angles

10° Cone

3” Gage

Open Face Volume = 11.21 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 0.976” | 1.359” | 0.718 |
| Center of round | 1.203” | 2.054” | 0.586 |
| Gauge | 0.895” | 2.448” | 0.366 |

Primary Cutting Structure:

3 Pattern; Forward Spiral; Non-redundant;

Back rakes:

Cone 		20-12.5°

Nose 		12.5°

Shoulder 	15°-27.5°

Gage		30°

Secondary Cutting Structure:

Backup cutters are 0.0284-.0256” under exposed

Backup cutters are trailing by one blade

Components:

03 - 22/32 Nozzle Sleeves

03 - 60 Series Nozzles

17 - 1313 Primary Cutters

06 - 1313 Secondary Cutters

15 - ½” Knuckles