2053A Ø8-1/2” 616-YR(BR)

Bit Body:

Ø8-1/2”

6 Blades

3 primary blades

3 secondary blades

60/65/55/60/55/65 Blade angles

7.5° Cone

2.5” Gage with 5 PDC inserts per blade

Open Face Volume = 26.998 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.298” | 1.177” | 1.103 |
| Center of round | 1.768” | 1.791” | 0.987 |
| Gauge | 1.324” | 1.877” | 0.705 |

Primary Cutting Structure:

3 Pattern start to position 9; Reverse Spiral; Backup cutters trail one blade and are 5° more durable; Back reaming cutters;

Back rakes:

Cone 		20°-15°

Nose 		15°

Shoulder 	15°-25°

Gage		30°

Components:

06 - 22/32 Nozzle Sleeves

06 - 60 Series Nozzles

29 - 1613 Primary Cutters

31 - 1313 Cutters (12 Trimmers)

30 – 1308 WP PDC