| Drill Bit Specifications: |  |
| --- | --- |
| Cutter Size (Primary, Sec, Back Ream) | 5/8”, 1/2”, 1/2” |
| Cutter Count (Total, Gage, Back Ream) | 53, 10, 6 |
| Blade Count | 6 |
| Gage Length (Inches) | 2.50 |
| Nozzle Count | 6 |
| Nozzle Size | 60 |
| JSA (Sq. Inches) | 14.26 |
| Max. TFA (Sq. Inches) | 2.23 |
| Operating Parameters: |  |
| Hydraulic Flow Rate (Max gpm) | 690 |
| Max. Weight on Bit (1,000 Lbs.) | 32 |
| Make-up Torque (ft/lbs) | 16,488 |
| Nominal Dimensions: |  |
| Overall Length (Inches) | 14.10 |
| Make Up Length (Inches) | 9.98 |
| Bit Breaker | SKB-21010 |
| Connection | Ø4-1/2" API Reg. |
| Approx. Weight (Lbs.) | 95 |

Standard Features:

- Large open face volume assists cuttings removal

- Six nozzles optimize blade cleaning and cuttings removal

- PDC wear inserts on each blade ensure the longevity of the gage.

- Premium grade cutters maximize ROP and Life

616-YR(BR)  Nomenclature Explained:

- 6  =  Number of Blades

- 16 =  Primary Cutter Size (16mm)

- YR =  Backup Cutters

- BR =  Back Reaming Cutters
*Interchangeable nozzles.