| Drill Bit Specifications: |  |  |
| --- | --- | --- |
| Cutter Size (Primary, Sec) | 5/8”,1/2” |  |
| Cutter Count (Total, Gage) | 55, 5, 5 |  |
| Blade Count | 5 |  |
| Gage Length (Inches) | 2.00 |  |
| Nozzle Count (Primary, Sec.) | 5 |  |
| Nozzle Series (Primary, Sec.) | 60 |  |
| JSA (Sq. Inches) | 14.08 |  |
| Max. TFA (Sq. Inches) | 1.85 |  |
| Operating Parameters: |  |  |
| Max. Hydraulic Flow Rate (GPM) | 255-670 |  |
| Max. Weight on Bit (1,000 Lbs.) | 50 |  |
| Make-up Torque (ft/lbs) | 16,488 |  |
| Nominal Dimensions: |  |  |
| Overall Length (Inches) | 14.10 |  |
| Make Up Length (Inches) | 10.04 |  |
| Bit Breaker | SKB-21010 |  |
| Connection | Ø4-1/2" API Reg. |  |
| Approx. Weight (Lbs.) | 95 |  |

Standard Features:

Short bit length improves steerability

Large open face volume assists cuttings removal

Five nozzles optimize blade cleaning and cuttings removal

Premium grade cutters maximize ROP and Life

PDC inserts for increased gage protection

516-YR(K)(BR) Nomenclature Explained:

| - • | - 5 | - = | - Number of blades |
| --- | --- | --- | --- |
| - • | - 16 | - = | - Primary Cutter Size (16mm) |
| - • | - YR | - = | - Double Row of Cutters |
| - • | - K | - = | - Bullets |
| - • | - BR | - = | - Back Reaming Cutters |