2070A Ø8-1/2” 516-YR(K)(BR)

Designed a generic 8-1/2" curve/lateral for South Texas. Modeled after the Halliburton 8-1/2" MMD55MM.

Features:

- Aggressive backrake scheme.

- Knuckles in the cone for ROP limiters and impact arrestors.

- PDC elements in the Wear Pad.

- Up reaming cutters.

Bit Body:

Ø8-1/2”

5 Blades

3 primary blades

2 secondary blades

69/67/86/69/69 Blade angles

15° Cone

2” Gage

Open Face Volume = 40.0 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.515” | 1.448” | 1.046" |
| Center of round | 2.173” | 1.897” | 1.145" |
| Gauge | 1.408” | 2.013” | .699" |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary; Knuckles are trailing one blade and set up to engage at .090” per rev. Back reaming cutters and 5 PDC in gage per blade.

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	20°-25°

Gage		30°

Components:

05 - 22/32" Nozzle Sleeves

05 - 60 Series Nozzles

16 - 1613 Primary Cutters

30 - 1313 Cutters

09- 1308 Cutters

06 - ½” PDC Knuckles

20 - ½” Wear Pad PDC’s