2074A Ø8-1/2” 513-SR(K)(BR)

2074 was designed for “The Rest of the World Fleet” with no specific application info. 2074 is designed to drill the curve and lateral.

Features:

- Durable backrake scheme to drill smoothly.

- Carbide knuckles in cone to help tool face control in curve and wear down to speed up in lateral.

- PDC elements in Wear Pad.

- Up reaming cutters.

Bit Body:

Ø8-1/2”

5 Blades

3 primary blades

2 secondary blades

68/73/63/70/86 Blade angles

7.5° Cone

2.0” Gage

Open Face Volume = 29.969 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.186” | 1.093” | 1.085 |
| Center of round | 1.705” | 1.388” | 1.228 |
| Gauge | 1.357” | 1.976” | 0.687 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary; Back reaming cutters and 5 PDC in gage per blade.

Back rakes:

Cone 		17.5°

Nose 		17.5°

Shoulder 	17.5-30°

Gage		35°

Components:

05 - 22/32 Nozzle Sleeves

05 - 60 Series Nozzles

40 - 1313 Cutters

6 – 3/8” TCI Knuckles

25 - ½” Wear Pad PDC’s