2075A Ø8-1/2” 611-SR(K)(BR)

2075 was designed for “The Rest of the World Fleet” with no specific application info. 2075 is designed to drill the curve and lateral.

Features:

- Durable backrake scheme to drill smoothly.

- Carbide knuckles in cone to help tool face control in curve and wear down to speed up in lateral.

- PDC elements in Wear Pad.

- Up reaming cutters.

Bit Body:

Ø8-1/2”

6 Blades

3 primary blades

3 secondary blades

55/65/55/60/65/60 Blade angles

7.5° Cone

2.0” Gage

Open Face Volume = 22.823in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.061” | 1.157” | .917 |
| Center of round | 1.451” | 1.598” | .908 |
| Gauge | 1.362” | 1.918” | .710 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary; Back reaming cutters and 5 PDC in gage per blade.

Back rakes:

Cone 		20°

Nose 		20°

Shoulder 	20-29°

Gage		30°

Components:

06 - 22/32 Nozzle Sleeves

06 - 60 Series Nozzles

38 - 1111 Cutters

12 - 1313 Cutters

6 – 3/8” TCI Knuckles

30 - ½” Wear Pad PDC’s