2077A Ø8-3/4” 611-SR(K)(BR)

2077 was designed for “The Rest of the World Fleet” with no specific application info. 2077 is designed to drill the curve and lateral.

Features:

- Durable backrake scheme to drill smoothly.

- Carbide knuckles in cone to help tool face control in curve and wear down to speed up in lateral.

- PDC elements in Wear Pad.

- Up reaming cutters.

Bit Body:

Ø8-3/4”

6 Blades

3 primary blades

3 secondary blades

55/65/55/60/65/60 Blade angles

7.5° Cone

2.0” Gage

Open Face Volume = 20.342in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.070” | 1.164” | .919 |
| Center of round | 1.533” | 1.621” | .945 |
| Gauge | 1.488” | 1.971” | .755 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary; Back reaming cutters and 5 PDC in gage per blade.

Back rakes:

Cone 		20°

Nose 		20°

Shoulder 	20-29°

Gage		30°

Components:

06 - 22/32 Nozzle Sleeves

06 - 60 Series Nozzles

39 - 1111 Cutters

12 - 1313 Cutters

6 – 3/8” TCI Knuckles

30 - ½” Wear Pad PDC’s