2079A Ø6-1/8” 413-SR(K)

Bit Body:

Ø6-1/8”

4 Blades

2 primary blades

95/95/85/85 Blade angles

10° Cone

2” Gage

Open Face Volume = 17.03 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.012" | .921” | 1.098 |
| Center of round | 1.443” | 1.155” | 1.249 |
| Gauge | 0.786” | 1.444” | 0.544 |

Primary Cutting Structure:

2 Pattern; Reverse Spiral; Non-redundant;

Back rakes:

Cone 		17.5°

Nose 		18°

Shoulder 	18°-27°

Gage		30°

Components:

04 - 18/32 Nozzle Sleeves

04 - 60 Series Nozzles

23 - 1313 Primary Cutters

8 - ½” Knuckles

8 - ½” Diamond Gage