2081A Ø8-3/4” 516-SR

Designed an aggressive 8-3/4" straight vertical for Black Creek & Unit Petroleum in SW Louisiana.

Features:

- Aggressive backrake scheme (1986 Backrakes).

- .015" Cutters

Bit Body:

Ø8-3/4”

5 Blades

3 primary blades

2 secondary blades

68/73/63/70/86 Blade angles

7.5° Cone

2” Gage

Open Face Volume = 26.77 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.195” | 1.098” | 1.088 |
| Center of round | 1.723” | 1.246” | 1.323 |
| Gauge | 1.483” | 1.491” | .995 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary;

Cone 		15°

Nose 		15°

Shoulder 	15°

Gage		30°

Components:

05 - 22/32" Nozzle Sleeves

05 - 60 Series Nozzles

26 - 1613 Primary Cutters

5 - 1313 Cutters

1- 1308 Cutters