| Drill Bit Specifications: |  |  |
| --- | --- | --- |
| Cutter Size (Primary, Sec) | 5/8”,1/2” |  |
| Cutter Count (Total, Gage) | 55,6 |  |
| Blade Count | 6 |  |
| Gage Length (Inches) | 3.00 |  |
| Nozzle Count (Primary, Sec.) | 6 |  |
| Nozzle Series (Primary, Sec.) | 60 |  |
| JSA (Sq. Inches) | 20.8 |  |
| Max. TFA (Sq. Inches) | 2.23 |  |
| Operating Parameters: |  |  |
| Max. Hydraulic Flow Rate (GPM) | 700 |  |
| Max. Weight on Bit (1,000 Lbs.) | 40 |  |
| Make-up Torque (ft/lbs) | 40,563 |  |
| Nominal Dimensions: |  |  |
| Overall Length (Inches) | *16.0 |  |
| Make Up Length (Inches) | *11.1 |  |
| Bit Breaker | SKB-21011 |  |
| Connection | Ø6-5/8" API Reg. |  |
| *Long tool joint available upon request (adds 6” to fishing neck, makeup and overall length, and 64 lbs to approximate weight). Approx. Weight (Lbs.) | *160 |  |

Standard Features:

Selectively positioned bullets for smooth drilling

Large open face volume assists cuttings removal

Six nozzles optimize blade cleaning and cuttings removal

Premium grade cutters maximize ROP and Life

616-YR(K) Nomenclature Explained:

| - • | - 6 | - = | - Number of blades |
| --- | --- | --- | --- |
| - • | - 16 | - = | - Primary Cutter Size (16mm) |
| - • | - YR+ | - = | - Backup Cutters |
| - • | - K | - = | - Bullets |
| - • | - 6 | - = | - Number of nozzles |