2093A Ø7-7/8” 419-SR

2093 was made for Big 6 in Liberty County to drill to 6000’ to 8000’ wells.  Offset bit record shows mill tooth roller cone bits at 50-55 ft/hr, expect 4 blade 19mm PDC bit to easily exceed ROP of 80 ft/hr so went with a more durable backrake scheme to mitigate torque problems.

Bit Body:

Ø7-7/8”

4 Blades

2 primary blades

2 secondary blades

95/95/85/85 Blade angles

15° Cone

2.75” Gage

Open Face Volume = 35.589 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.343” | 1.080” | 1.244 |
| Center of round | 1.758” | 1.432” | 1.228 |
| Gauge | 1.053” | 2.157” | 0.488 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary;

Back rakes:

Cone 		17.5°

Nose 		17.5°

Shoulder 	17.5-30°

Gage		35°

Components:

04 - 22/32 Nozzle Sleeves

04 - 60 Series Nozzles

14 - 1913 Primary Cutters

05 - 1613 Cutters