2095A Ø9-7/8” 616-YR(K)

2095 was made for Oxy in the Permian to drill on a motor with a Weatherford RSS.

Features:

- Uses the same cutter set as a 2063.

- 6” Step Gage: 5” at gage, 1” (on tool joint side) 1/32” under gage on the diameter.

Bit Body:

Ø9-7/8”

6 Blades

3 primary blades

6 secondary blades

55/60/55/65/60/65 Blade angles

12.5° Cone

6” Gage

Open Face Volume = 28.06 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.072” | 1.487” | .720” |
| Center of round | 1.663” | 1.773” | .938” |
| Gauge | 1.461” | 1.836” | .796" |

Primary Cutting Structure:

Redundant; Reverse Spiral; Secondary blades are underexposed by -.050" to primary; Knuckles are trailing by one blade.

Back rakes:

Cone 		15°

Nose 		17.5°

Shoulder 	20°-25°

Gage		30°

Components:

06 - 22/32" Nozzle Sleeves

06 - 60 Series Nozzles

24 - 1613 Primary Cutters

36 - 1313 Cutters

10 - ½” PDC Knuckles