2096A Ø6-3/4” 513-XR(K)

2096 was made for Oxy in the Permian to drill on a motor with a Weatherford RSS.

Features:

- Backups placed at 380 ft/hr @ 190 rpm (.4”/rev).

- 1313 Primary Cutters with 1111 backup cutters.

- 4” Step Gage: 3.375” at gage, .625” (on tool joint side) 1/32” under gage on the diameter.

Bit Body:

Ø6-3/4”

5 Blades

3 primary blades

2 secondary blades

68/73/63/70/86 Blade angles

15° Cone

4” Gage

Open Face Volume = 10.488 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.070” | 1.189” | 0.900 |
| Center of round | 1.367” | 1.716” | 0.797 |
| Gauge | 1.293” | 2.061” | 0.627 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary; Backup cutters are set at .400”/rev under exposed.

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15-22.5°

Gage		35°

Components:

05 - 16/32 Nozzle Sleeves

05 - 55 Series Nozzles

27 - 1313 Primary Cutters

10 - 1111 Cutters

01 - 1308 Cutters

03 – 3/8” TCI Bullets