2110A Ø9-7/8” 516-XR(K)

2110 was made for Oxy in the Permian. Made to look like Smith’s SSi516PDC. Oxy dropped Weatherford, and the new design to work with the low WOB and RSS 2104, will not be used now. Needed a bit to work with high WOB.

Features:

- Aggressive backrake in cone. Liner change to durable backrake on shoulder to gage.

- backups engage at .833”/rev (500ft/hr, 120rpm)

- 4” Gage.

Bit Body:

Ø9-7/8”

5 Blades

3 primary blades

2 secondary blades

65/70/80/75/70 Blade angles

15° Cone

4” Gage

Open Face Volume = 50.445 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.462” | 1.282” | 1.140” |
| Center of round | 2.060” | 1.847” | 1.115” |
| Gauge | 1.432” | 2.532” | 0.566" |

Primary Cutting Structure:

Single set; Reverse Spiral;

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	16°-30°

Gage		35-40°

Components:

05 - 22/32" Nozzle Sleeves

05 - 60 Series Nozzles

30 - 1613 Primary Cutters

24 - 1610 Cutters

21 - ½” TCI Knuckles