2114A Ø12-1/4” 619-XR9(K)

2114 was made for Marathon in Karnes County, to drill 110’ to 5400’ wells on Performance Motor. 260 rpm, 1100 gal/min, expected ROP of 400 ft/hr.

Features:

- Aggressive Backrake.

Bit Body:

Ø12-1/4”

6 Blades

3 primary blades

3 secondary blades

55/60/55/65/60/65 Blade angles

15° Cone

3” Gage

Open Face Volume = 80.630 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.602” | 1.295” | 1.237 |
| Center of round | 2.550” | 1.880” | 1.356 |
| Gauge | 2.241” | 2.976” | 0.753 |

Primary Cutting Structure:

Redundant; Reverse Spiral;

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15-30°

Gage		35°

Components:

09 - 22/32 Nozzle Sleeves

09 - 60 Series Nozzles

43 - 1913 Primary Cutters

12 - 1/2” TCI Knuckles