| Drill Bit Specifications: |  |  |
| --- | --- | --- |
| Cutter Size (Primary, Sec) | 5/8”,1/2” |  |
| Cutter Count (Total,Gage,Back Ream) | 39,5,5 |  |
| Blade Count | 5 |  |
| Gage Length (Inches) | 3.00 |  |
| Nozzle Count (Primary, Sec.) | 5 |  |
| Nozzle Series (Primary, Sec.) | 55 |  |
| JSA (Sq. Inches) | 9.64 |  |
| Max. TFA (Sq. Inches) | 0.98 |  |
| Operating Parameters: |  |  |
| Max. Hydraulic Flow Rate (GPM) | 306 |  |
| Max. Weight on Bit (1,000 Lbs.) | 22 |  |
| Make-up Torque (ft/lbs) | 7,611 |  |
| Nominal Dimensions: |  |  |
| Overall Length (Inches) | 12.26 |  |
| Make Up Length (Inches) | 8.51 |  |
| Bit Breaker | SKB-21012 |  |
| Connection | Ø3-1/2" API Reg. |  |
| Approx. Weight (Lbs.) | 60 |  |

Tailor-Made Features:

Premium grade cutters maximize ROP and Bit Life

Customized gauge design featuring 3” total gauge length - 2” passive gauge plus 1” relieved gauge

Nozzle configuration and large open face volume optimize blade cleaning and cuttings removal

516-XR(BR) Nomenclature Explained:

| - • | - 5 | - = | - Number of Blades |
| --- | --- | --- | --- |
| - • | - 16 | - = | - Primary Cutter Size (16mm) |
| - • | - XR+ | - = | - Backup Cutters |
| - • | - BR | - = | - Back Reaming Cutters |