2117A Ø6-3/4” 516-XR(BR)

2115 was made for Oxy in the Permian to drill on a motor with a RSS, customer wanted a 16mm bit (had a 13mm 2096) based on success of a Varel 516. 2117 was made because they wanted a longer gage.

Features:

- Backups placed at 400 ft/hr @ 200 rpm (.4”/rev).

- 1613 Primary Cutters with 1313 backup cutters.

- 4.8” Step Gage: 4” at gage, .8” (on tool joint side) 1/32” under gage on the diameter.

Bit Body:

Ø6-3/4”

5 Blades

3 primary blades

2 secondary blades

65/70/80/75/70 Blade angles

10° Cone

4.8” Gage

Open Face Volume = 10.062 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.094” | 1.249” | 0.876 |
| Center of round | 1.367” | 1.849” | 0.739 |
| Gauge | 1.293” | 2.062” | 0.627 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary; Backup cutters are set at .400”/rev under exposed.

Back rakes:

Cone 		12.5°

Nose 		12.5°

Shoulder 	12.5-30°

Gage		35°

Components:

05 - 16/32 Nozzle Sleeves

05 - 55 Series Nozzles

19 - 1613 Primary Cutters

20 - 1313 Cutters