2119A Ø12-1/4” 614.5-SR+9(K)

2119 was made for Endeavor in West Texas, to drill 1325’ to 9200’ wells on Performance Motor. 112rpm @ 100 ft/hr. Offsets were ≈200rpm, customer is committed to run more weight (starting at 40k) and less rpm to reach the target of just one bit for the whole interval (currently taking 3-4bits).

Features:

- 14.5mm Cutters.

Bit Body:

Ø12-1/4”

6 Blades

3 primary blades

3 secondary blades

55/60/55/65/60/65 Blade angles

15° Cone

3” Gage

Open Face Volume = 70.411 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.602” | 1.515” | 1.057 |
| Center of round | 2.550” | 2.091” | 1.220 |
| Gauge | 2.241” | 3.051” | 0.735 |

Primary Cutting Structure:

Redundant; Reverse Spiral;

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15-30°

Gage		35°

Components:

09 - 22/32 Nozzle Sleeves

09 - 60 Series Nozzles

49 - 14.513 Primary Cutters

09 - 1613 Primary Cutters

18 - 1313 Primary Cutters

06 - 1/2” TCI Knuckles

15 - 1/2” PDC Knuckles