2120A Ø8-3/4” 517-SR7(K)(BR)

2120 was made for Shell to drill 4750’-9700’ with .25rev/gal motor at 586 gal/min and top drive rpm of 70 @ 95+ ft/ hr (backups placed at 0.229”/rev, 200ft/hr@175rpm). This is the fast top hole bit.

Features:

- Aggressive backrake in cone.

- 17mm cutters in cone to nose.

Bit Body:

Ø8-3/4”

5 Blades

3 primary blades

2 secondary blades

65/70/80/75/70 Blade angles

12.5° Cone

2” Gage

Open Face Volume = 34.571 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.391” | 1.218” | 1.142” |
| Center of round | 1.969” | 1.837” | 1.072” |
| Gauge | 1.481” | 2.319” | 0.639" |

Primary Cutting Structure:

Single set; Reverse Spiral;

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15°-30°

Gage		35°

Components:

07 - 22/32" Nozzle Sleeves

07 - 60 Series Nozzles

11 - 1713 Primary Cutters

15 - 1613 Cutters

10 - 1313 Cutters

10 – ½” PDC Bullets