Basic Application Definition required for Design Request

1. Customer name – Shell E&P

2. County – Loving County, Texas

3. Bit Size – 8.75” CR517 Double Row (SR9)

4. Measured Depth In, Measured Depth Out – 4750’ +/- 9,700’

5. Motor? Straight or bent housing? Revs per Gallon? Yes, Straight .25RPG

6. Gallons per Minute – 586 (550-600)

7. Surface RPM – 70

8. Expected ROP – 95 ft/hr

9. Specs or Photos for current bits of choice – Halliburton HDBS MMD65DM or MM65DM Spec Sheets

10. Indication of dull condition on current bits– photos best, dull grades if no photos –

11. Formation Tops and characteristics (compressive strength) – Included Loving County Sparta

12. Bit Records that objectively show previous drilling parameters and results – Included Bit Records and Sparta Analysis (HDBS Rock Strength Analysis).

13. Customer’s stated targets (ROP, footage, dull condition), and any customer preferences for bit features or operating parameters that will be difficult to sell around.

*Customer is very interested in 17mm Cutter for 5 Blade with backups on shoulder to speed up ROP down to pull point around 9,700 where he will make a short trip and pickup sharp bit to finish interval down to TD as economics are in his favor with fast-tripping efficiency of Nabors 322.

14. Date needed –  9/30/15