2121A Ø9-7/8” 516-XR(BR)

2109 was made for Unit to drill 3000’-12000’ with .29rev/gal motor at 500 gal/min and top drive rpm of 80 @ 150 ft/ hr (backups placed at 0.133”/rev, 150ft/hr@225rpm). This is to displace the MM55DM.

2109 had high erosion on the front and back of the blades. 2121 was made to combat the erosion problem. The center two nozzles were removed, the front of the blades was given a taper (so still have full capture in the back of the pocket), new hard metal will be used everywhere, and back reaming cutters were added.

Features:

- aggressive backrake to work with low WOB

- backups engage at .182”/rev

Bit Body:

Ø9-7/8”

5 Blades

3 primary blades

2 secondary blades

68/73/63/70/86 Blade angles

15° Cone

3” Gage

Open Face Volume = 44.961 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.335” | 1.330” | 1.004” |
| Center of round | 1.842” | 1.910” | 0.964” |
| Gauge | 1.430” | 2.465” | 0.580" |

Primary Cutting Structure:

Single set; Reverse Spiral;

Back rakes:

Cone 		15°

Nose 		15°-12.5°

Shoulder 	12.5°-17°

Gage		30°

Components:

05 - 22/32" Nozzle Sleeves

05 - 60 Series Nozzles

28 - 1613 Primary Cutters

25 - 1313 Cutters

1 - 1308 Cutter