| Drill Bit Specifications: |  |  |
| --- | --- | --- |
| Cutter Size (Primary, Sec) | 5/8", 1/2" |  |
| Cutter Count (Total, Gage) | 65,10 |  |
| Blade Count | 7 |  |
| Gage Length (Inches) | 3.00 |  |
| Nozzle Count (Primary, Sec.) | 7 |  |
| Nozzle Series (Primary, Sec.) | 60 |  |
| JSA (Sq. Inches) | 32.92 |  |
| Max. TFA (Sq. Inches) | 2.60 |  |
| Operating Parameters: |  |  |
| Max. Hydraulic Flow Rate (GPM) | 810 |  |
| Max. Weight on Bit (1,000 Lbs.) | 60 |  |
| Make-up Torque (ft/lbs) | 40,563 |  |
| Nominal Dimensions: |  |  |
| Overall Length (Inches) | *17.87 |  |
| Make Up Length (Inches) | *13.00 |  |
| Bit Breaker | SKB-21011 |  |
| Connection | Ø6-5/8" API Reg. |  |
| *Long tool joint available upon request (adds 6” to fishing neck, makeup and overall length, and 64 lbs to approximate weight). Approx. Weight (Lbs.) | *220 |  |

Standard Features:

Premium grade cutters maximize ROP and Life

Large open face volume assists cuttings removal

Seven nozzles optimize blade and cuttings removal

713-SR(K) Nomenclature Explained:

| - • | - 7 | - = | - Number of blades |
| --- | --- | --- | --- |
| - • | - 13 | - = | - Primary Cutter Size (13mm) |
| - • | - SR | - = | - Single Row Cutters |
| - • | - 7 | - = | - Number of nozzles |

- *Interchangeable nozzles.