2122A Ø12-1/4” 713-SR(K)

2122 was designed to drill depth in ~7000' to depth out ~8000' at 20 ft/hr for Vitruvian in Oklahoma

Features:

- 7 blades

- Bullets on shoulder for DBR breaks.

- 16mm aggressive backrakes in the cone

- 13mm out on the shoulder

Bit Body:

Ø12-1/4”

7 Blades

3 primary blades

4 secondary blades

46/50/46/54/58/50/56 Blade angles

15° Cone

3” Gage

Open Face Volume = 56.008 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.545” | 1.476” | 1.047 |
| Center of round | 1.983” | 2.079” | 0.954 |
| Gauge | 2.257” | 2.762” | 0.817 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary; knuckles are set at .091”/rev.

Back rakes:

Cone 		12.5°

Nose 		12.5°

Shoulder 	15-25°

Gage		30°

Components:

07 - 22/32 Nozzle Sleeves

07 - 60 Series Nozzles

16 - 1613 Cutters

49 - 1313 Cutters

28 - ½” PDC Knuckles