| Drill Bit Specifications: |  |  |
| --- | --- | --- |
| Cutter Size (Primary, Sec) | 5/8”,1/2” |  |
| Cutter Count (Total, Gage, Back Ream) | 83,7,5 |  |
| Blade Count | 5 |  |
| Gage Length (Inches) | 6.00 |  |
| Nozzle Count (Primary, Sec.) | 8 |  |
| Nozzle Series (Primary, Sec.) | 60 |  |
| JSA (Sq. Inches) | 42.33 |  |
| Max. TFA (Sq. Inches) | 2.97 |  |
| Operating Parameters: |  |  |
| Max. Weight on Bit (1,000 Lbs.) | 60 |  |
| Make-up Torque (ft/lbs) | 40,563 |  |
| Nominal Dimensions: |  |  |
| Overall Length (Inches) | *22.86 |  |
| Make Up Length (Inches) | *17.99 |  |
| Bit Breaker | SKB-21011 |  |
| Connection | Ø6-5/8" API Reg. |  |
| *Long tool joint available upon request (adds 6” to fishing neck, makeup and overall length, and 64 lbs to approximate weight). Approx. Weight (Lbs.) | *285 |  |

Tailor-Made Features:

Premium grade cutters maximize ROP and Life

Customized step gauge design

Eight nozzles optimize blade cleaning and cuttings removal

Large open face volume assists cuttings removal

PDC inserts for increased gage protection

516-XR8(K)(BR) Nomenclature Explained:

| - • | - 5 | - = | - Number of blades |
| --- | --- | --- | --- |
| - • | - 16 | - = | - Primary Cutter Size (16mm) |
| - • | - XR+ | - = | - Backup Cutters |
| - • | - 8 | - = | - Number of Nozzles |
| - • | - K | - = | - Bullets |
| - • | - BR | - = | - Back Reaming Cutters |