2124A Ø12-1/4” 516-XR8(K)(BR)

2124 is a 2082 with the backup elements lowered. 400ft/hr @ 200rpm = .4”/rev. Made to drill 3000’ of salt.

Features:

- Bullets on nose for ROP limiters and impact arrestors.

- Three carbide bullets on gage to absorb lateral impacts.

- Elliptical profile to get more diamonds on the shoulder.

- Step gage, 4” at gage, last 2” 1/23” under on diameter (tool joint side).

Bit Body:

Ø12-1/4”

5 Blades

3 primary blades

2 secondary blades

68/73/63/70/86 Blade angles

15° Cone

6” Gage

Open Face Volume = 56.008 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.545” | 1.476” | 1.047 |
| Center of round | 1.983” | 2.079” | 0.954 |
| Gauge | 2.257” | 2.762” | 0.817 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary; Backup cutters are set at .4”/rev.

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15-20°

Gage		40°

Components:

08 - 22/32 Nozzle Sleeves

08 - 60 Series Nozzles

49 - 1613 Primary Cutters

33 - 1313 Cutters

01 - 1308 Cutter

11 - ½” PDC Knuckles

15 - ½” TCI Knuckles

25 - Wear Pad PDC