2126A Ø6-3/4” 511-SR(BR)

2126A was made for Oxy to drill the lateral on a motor with a RSS. Current bit of choice is the Varel 511 (occasionally a 613 is used), so customer wanted a 511 bit (even though this has more 13’s). 2126 was made with 11’s in the cone and 13’s on the shoulder with the intent of speeding up the bit.

Features:

- 1111 Primary Cutters in cone to nose, with 1313 cutters from nose to gage.

- 6” Taper Gage: 2” at gage, 4” (on tool joint side) .6° Taper (.0419 over 4 in. under gage on the radius).

Bit Body:

Ø6-3/4”

5 Blades

3 primary blades

2 secondary blades

65/70/80/75/70 Blade angles

12.5° Cone

6” Gage

Open Face Volume = 13.538 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.081” | 0.837” | 1.292 |
| Center of round | 1.367” | 1.429” | 0.957 |
| Gauge | 1.293” | 1.914” | 0.676 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary;

Back rakes:

Cone 		12.5°

Nose 		12.5°

Shoulder 	12.5-30°

Gage		35°

Components:

05 - 16/32 Nozzle Sleeves

05 - 55 Series Nozzles

09 - 1111 Primary Cutters

24 - 1313 Cutters