2131A Ø7-7/8” 616-SR(K)

2131 was made to lower cutter cost of the 2031 by replacing the backup cutters with bullets. The bit was also changed to a single set from a redundant, and the backrake on the outer shoulder was made more durable.

Features:

- Bullets set to engage at 20 ft/hr at 200 rpm after .0625” of primary cutter is missing.

Bit Body:

Ø7-7/8”

6 Blades

3 primary blades

3 secondary blades

60/55/65/60/65/55 Blade angles

15° Cone

2” Gage

Open Face Volume = 19.49 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 0.905” | 1.173” | 0.772 |
| Center of round | 1.285” | 1.644” | 0.782 |
| Gauge | 0.923” | 1.828” | 0.505 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary.

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15-30°

Gage		35°

Components:

06 - 22/32 Nozzle Sleeves

06 - 60 Series Nozzles

07 - 1313 Primary Cutters

26 - 1613 Cutters

12 – ½” PDC Bullets