| Drill Bit Specifications: |  |  |
| --- | --- | --- |
| Cutter Size (Primary, Sec) | 3/4” |  |
| Cutter Count (Total, Gage) | 33,5 |  |
| Blade Count | 5 |  |
| Gage Length (Inches) | 3.00 |  |
| Nozzle Count (Primary, Sec.) | 8 |  |
| Nozzle Series (Primary, Sec.) | 60 |  |
| JSA (Sq. Inches) | 30.08 |  |
| Max. TFA (Sq. Inches) | 2.97 |  |
| Operating Parameters: |  |  |
| Max. Weight on Bit (1,000 Lbs.) | 60 |  |
| Make-up Torque (ft/lbs) | 40,563 |  |
| Nominal Dimensions: |  |  |
| Overall Length (Inches) | *17.25 |  |
| Make Up Length (Inches) | *12.38 |  |
| Bit Breaker | SKB-21011 |  |
| Connection | Ø6-5/8" API Reg. |  |
| *Long tool joint available upon request (adds 6” to fishing neck, makeup and overall length, and 64 lbs to approximate weight). Approx. Weight (Lbs.) | *185 |  |

Standard Features:

Large open face volume assists cuttings removal

Eight nozzles optimize blade cleaning and cuttings removal

Premium grade cutters maximize ROP and Life

519-SR8 Nomenclature Explained:

| - • | - 5 | - = | - Number of blades |
| --- | --- | --- | --- |
| - • | - 19 | - = | - Primary Cutter Size (19mm) |
| - • | - SR+ | - = | - Single Row of Cutters |
| - • | - 8 | - = | - Number of Nozzles |