2141A Ø12-1/4” 616-XR(K)(BR)

2141 was designed to drill from 5500-8500' in Midland & Ector County for Callon. The bit was made more durable than the 2134 currently in use in an attempt to TD the well in one run after the drill-out bit was pulled. 180 rpm at 225 ft/hr was the intended goal. Tried to reduce the number of cutters and length of the 2134 to reduce cutter cost. Took the "old profile" shape and the long elliptical profile and made it in between. Shorten the 4" gage length to 3". Set the knuckles and backup elements to DOC of .25"

Features:

- Medium backrake

- backups engage at .25”/rev (180 rpm at 225 ft/hr)

- Non redundant

Bit Body:

Ø12-1/4”

6 Blades

3 primary blades

6 secondary blades

55/60/55/65/60/65 Blade angles

15° Cone

3” Gage

Open Face Volume = in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.227" | 1.470" | .83 |
| Center of round | 1.412" | 1.759" | .80 |
| Gauge | 1.691" | 2.039" | .83 |

Primary Cutting Structure:

Redundant; Reverse Spiral; Secondary blades are equally exposed to primary;

Back rakes:

Cone 		17.5°

Nose 		15°

Shoulder 	15°-25°

Gage		30°

Components:

09 - 22/32" Nozzle Sleeves

09 - 60 Series Nozzles

48 - 1613 Primary Cutters

31 - 1313 Cutters