2142A Ø8-3/4” 616-SR(K)

2142 is a "scaled" version of the 2140. The bit is a single set reverse spiral, with durable back rake in the cone to help with the impact damage that has been happening to 2140. nose and shoulder cutters have aggressive 15° backrake. The blade angles and nozzles are the same as 2140. It also shares the smae tapered 3" gage as the 2140.

Features:

- Bullets set to engage at .167 in DOC.

Bit Body:

Ø8-3/4”

6 Blades

3 primary blades

3 secondary blades

60/55/65/60/65/55 Blade angles

15° Cone

3” Gage

Open Face Volume =  in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.103 | 1.382 | .799 |
| Center of round | 1.633 | 1.730 | .944 |
| Gauge | 1.245 | 1.770 | .703 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary.

Back rakes:

Cone 		18°

Nose 		15°

Shoulder 	15-29°

Gage		30°

Components:

06 - 22/32 Nozzle Sleeves

06 - 60 Series Nozzles

34 - 1613 Cutters

6 – ½” PDC Bullets

3-7/16" PDC Bullets