2148 Ø12-1/4” 616-XR9(K)(BR)

Oxy was in need of a 12-1/4" with the longest gage possible. Took the 2138 and made the gage length 8.25":

- 2 bit interval, 2078 will be the top bit. (More fast than durable.)

- .020” Chamfers on the cutters.

- Carbide bullets just above the gage.

- Trimmers set at a durable backrake, aggressive backrake elsewhere.

- 6” Gage with the 2073 profile.

- Backup cutters underexposed .4"/rev, with a 10° more durable backrake.

- PDC bullets will be underexposed in the cone.

- Smaller inner profile diameter.

Features:

- Bullets in cone to nose for ROP limiters and impact arrestors.

- Three carbide bullets on gage to absorb lateral impacts.

- Elliptical profile to get more diamonds on the shoulder.

Bit Body:

Ø12-1/4”

6 Blades

3 primary blades

3 secondary blades

55/60/55/65/60/65 Blade angles

15° Cone

6” Gage

Open Face Volume = 51.319 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.545” | 1.475” | 1.047 |
| Center of round | 1.983” | 1.855” | 1.069 |
| Gauge | 2.257” | 2.33” | 0.969 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary; Knuckles are under exposed in the cone and nose at .025”; all other backup elements are even exposure with the primary elements.

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15-20°

Gage		40°

Components:

09 - 22/32 Nozzle Sleeves

09 - 60 Series Nozzles

56 - 1613 Primary Cutters

42 - 1313 Cutters

12 - ½” PDC Knuckles

18 - ½” TCI Knuckles

30 - Wear Pad PDC