2153 Ø12-1/4” 713-XR+(K)(BR)

Parsley needs a bit that will drill the curve from 8600' to 9100' @ 30ft/hr through the SprayberrySand/Shale/Lime. Their current bits of choice are rock bits (Smith GF15B, SEC GTD64C, BHI Kymera):

- 1 Bit run per repair

- DP cutters on the shoulder

- Active gage trimmers to build the curve

- Durable backrake.

- 3” Gage 2" full and 1" taper (.016" per side)

- Carbide knuckles are underexposed on the trimmers by .050"

- Carbide knuckles are flush with the OD grind of the wear pads

- Backup cutters are at a DOC of .167"

- 1313 back up cutters.

Features:

- Bit Body:

Ø12-1/4”

7 Blades

3 primary blades

4 secondary blades

46/50/46/54/58/50/56 Blade angles

7° Cone

3” Gage

Open Face Volume =  in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. |  |  |  |
| Center of round |  |  |  |
| Gauge |  |  |  |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary;

Back rakes:

Cone 		17.5°

Nose 		15°

Shoulder 	15-30°

Gage		40°

Components:

07 - 22/32 Nozzle Sleeves

07 - 60 Series Nozzles

89 - 1613 Primary Cutters

15 - 1313 Cutters

09 - ½” PDC Knuckles

14 - ½” TCI Knuckles

35 - Wear Pad PDC