| Drill Bit Specifications: |  |  |
| --- | --- | --- |
| Cutter Size (Primary, Sec) | 5/8”,1/2” |  |
| Cutter Count (Total, Gage) | 28,6 |  |
| Blade Count | 5 |  |
| Gage Length (Inches) | 3.00 |  |
| Nozzle Count (Primary, Sec.) | 5 |  |
| Nozzle Series (Primary, Sec.) | 60 |  |
| JSA (Sq. Inches) | 10.84 |  |
| Max. TFA (Sq. Inches) | 1.86 |  |
| Operating Parameters: |  |  |
| Max. Weight on Bit (1,000 Lbs.) | 60 |  |
| Make-up Torque (ft/lbs) | 17,399 |  |
| Nominal Dimensions: |  |  |
| Overall Length (Inches) | 14.61 |  |
| Make Up Length (Inches) | 10.49 |  |
| Bit Breaker | SKB-21010 |  |
| Connection | Ø4-1/2" API Reg. |  |
| Approx. Weight (Lbs.) | 95 |  |

Standard Features:

Premium grade cutters maximize

- ROP and Life

Large open face volume assists cuttings removal

Five nozzles optimize blade cleaning and cuttings removal

516-SR(K) Nomenclature Explained:

5 = Number of Blades

16 = Primary Cutter Size (16mm)

SR - Single Row of Cutters

K = Bullets