2152A Ø7-7/8” 516-SR(K)

2152 was designed for XTO, in WTNM. For use on a motor, from 1,700’ to 7,500’. Target ROP is 110’/hr over the whole interval. 2152 is based on 2131 with the bullets moved into the cone and one blade removed. The cone backrakes were made more aggressive (15° to 12.5°). Cutter spacing was reduced in the cone to increase cutter density and durability.

Features:

- Bullets set to engage at 360 ft/hr at 180 rpm (.4”/rev).

Bit Body:

Ø7-7/8”

5 Blades

3 primary blades

2 secondary blades

65/70/80/75/70 Blade angles

12.5° Cone

3” 1° Tapered Gage

Open Face Volume = 22.372 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.027” | 1.438” | 0.714 |
| Center of round | 1.416” | 1.693” | 0.836 |
| Gauge | 0.920” | 1.746” | 0.527 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary.

Back rakes:

Cone 		12.5°

Nose 		15°

Shoulder 	15-30°

Gage		35°

Components:

05 - 22/32 Nozzle Sleeves

05 - 60 Series Nozzles

06 - 1313 Primary Cutters

22 - 1613 Cutters

06 – 7/16” PDC Bullets