CALLON PETROLEUM COMPANY

GEOLOGICAL PROGNOSIS

Casselman40 #10UL

Surface Location: Sec 40, ABST 1139, T.&P. & R.R. Co. Survey

400’ FSL / 1964’ FEL

Bottom Hole Location: Sec 40, ABST 1139, T.&P. & R.R. Co. Survey

100’ FNL / 1515’ FEL

| Midland County |  |  |  |
| --- | --- | --- | --- |
| Casselman40 #10UL |  |  |  |
| Formation |  | Top (TVD) | Subsea (KB~2901`) |
| Yates | 2980 |  | -79 |
| Queen | 4105 |  | -1204 |
| Grayburg | 4550 |  | -1649 |
| San Andres | 5175 |  | -2274 |
| Clearfork | 7105 |  | -4204 |
| Spraberry | 8110 |  | -5209 |
| Lower Spraberry LSBY Shale | 8625 9036 |  | -5724 -6135 |
| Upper LSBY LZ | 9077 |  | -6176 |
| Dean | 9350 |  | -6449 |
| Wolfcamp Wolfcamp B Top | 9502 |  | -6601 |
| Wolfcamp B Base |  |  |  |
| WFMP LWR B LZ WFMP C LZ |  |  |  |

MUD Logging:  ALS

Mud logging crew will be rigged up and ready to catch samples at drill out of intermediate casing (approx 0’).  30’ samples will be collected and described to TD.  Wet samples can be discarded in pit.

Mud logger will notify geologist in all instances of any unusual occurrences (e.g. oil-on-pits) and significant oil and gas shows.  A complete mud log report, with brief summary will be emailed to geologist twice a day.

GEOSTEERING:  Terra Guidance

ELECTRIC LOGS:	MS Guidance - GR MWD BHA at drill out of intermediate casing.

** Callon Petroleum 2017 **

CALLON PETROLEUM CONTACTS

GEOLOGIST:

Jake Lacy

Cell Phone:	281-799-3478

Office:		281-589-5271

Email:		jlacy@callon.com

Josh Peacock

Cell Phone: 	281-352-6411

Office:		281-589-5263

Email:		jpeacock@callon.com

DRILLING ENGINEER:

Russ Truby

Cell: 		832-289-2741

Office: 		281-589-5277

Email: 		rtruby@callon.com

David Newberry

Cell:		832-520-7720

Office:		281-589-5275

Email:		dnewberry@callon.com

DISTRIBUTION:		See below for Callon distribution. List below does not include any partner distribution that may

be required.

** Callon Petroleum 2017 **