2158A Ø11” 616-SR9(K)

2158 is more 6-3 than the 2030, with the same backrake scheme. More aggressive cone by increased spacing in cone and first redundant removed. Gage has been increased to 4”. Wider blade to prevent blade cracks and to hold TCI knuckles in the cone.

Bit Body:

Ø11”

6 Blades

3 primary blades

3 secondary blades

55/60/55/65/60/65 Blade angles

15° Cone

4” Gage

Open Face Volume = 60.819 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.402” | 1.512” | 0.927 |
| Center of round | 2.028” | 1.965” | 1.032 |
| Gauge | 1.609” | 2.248” | 0.716 |

Primary Cutting Structure:

3 Pattern start to position 9; Reverse Spiral;

Secondary blades are under exposed by .083”;

Back rakes:

Cone 		15°-12.5°

Nose 		12.5°

Shoulder 	12.5°-27.5°

Gage		30°

Components:

09 - 22/32 Nozzle Sleeves

09 - 60 Series Nozzles

42 - 1613 Primary Cutters

7   - 1313 Cutters

6   - ½” TCI Knuckles