2160A Ø17-1/2” 616-SR+9(K)

2160 uses a new 1018 material for the donut plus new welding procedure to eliminate cracks in the donut weld. 2097 was a nozzle tweak to the 2085. 2085 was made to drill to 1700’ wells in West Texas on a motor with a 1.5° bend. To increase durability partial backup design was chosen with PDC Bullets.

Features:

- Bullets on nose and outer shoulder for ROP limiters and impact arrestors.

- Three 1313 backups per blade.

- 9” gage

Bit Body:

Ø17-1/2”

6 Blades

3 primary blades

3 secondary blades

55/60/55/65/60/65 Blade angles

15° Cone

9” Gage

Open Face Volume = 305.119 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 2.538” | 1.485” | 1.709 |
| Center of round | 3.506” | 2.411” | 1.454 |
| Gauge | 2.726” | 2.937” | 0.928 |

Primary Cutting Structure:

Redundant; Reverse Spiral; Secondary blades are flush to primary; Backup cutters are set at .093”/rev +.05” under exposed. Bullets are set at .093”/rev +.025” under exposed

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15-20°

Gage		30°

Components:

09 - 22/32 Nozzle Sleeves

09 - 60 Series Nozzles

69 - 1613 Primary Cutters

31 - 1313 Cutters

30 - ½” PDC Knuckles