| Drill Bit Specifications: |  |  |
| --- | --- | --- |
| Cutter Size (Primary, Sec) | 5/8”,1/2” |  |
| Cutter Count (Total, Gage, Back Ream) | 61,10,5 |  |
| Blade Count | 5 |  |
| Gage Length (Inches) | 3.00 |  |
| Nozzle Count (Primary, Sec.) | 8 |  |
| Nozzle Series (Primary, Sec.) | 60 |  |
| JSA (Sq. Inches) | 26.58 |  |
| Max. TFA (Sq. Inches) | 2.97 |  |
| Operating Parameters: |  |  |
| Max. Hydraulic Flow Rate (GPM) | 650 |  |
| Max. Weight on Bit (1,000 Lbs.) | 60 |  |
| Make-up Torque (ft/lbs) | 17,399 |  |
| Nominal Dimensions: |  |  |
| Overall Length (Inches) | 16.79 |  |
| Make Up Length (Inches) | 12.73 |  |
| Bit Breaker | SKB-21010 |  |
| Connection | Ø4-1/2" API Reg. |  |
| Approx. Weight (Lbs.) | 140 |  |

Standard Features:

Premium grade cutters maximize ROP and Life

Large open face volume assists cuttings removal

Eight nozzles optimize blade cleaning and cuttings removal

Selectively positioned bullets for smooth drilling

PDC inserts for increased gage protection

1" full gage, 2" tapered gage at 1°

516-XR8(K)(BR) Nomenclature Explained:

| - • | - 5 | - = | - Number of Blades |
| --- | --- | --- | --- |
| - • | - 16 | - = | - Primary Cutter Size (16mm) |
| - • | - XR+ | - = | - Backup Cutters |
| - • | - 8 | - = | - Number of Nozzles |
| - • | - K | - = | - Bullets |
| - • | - BR+ | - = | - Back Reaming Cutters |