2164B Ø9-7/8” 516-XR(K)(BR)

2164 is for Jagged Peak to drill the curve from 9200' to 10200' with a .26 motor @ 600 gal/min with an ROP between 30-40 ft/hr. The design has a flat profile (5°) with an active gage. The gage length is 3" (1" full and 2" tapered at 1°). The 1" full gage will have 2 PDC elements per pad. Durable back-rakes (20° all across the board) with diamond knuckles in the cone and carbide knuckles behind the trimmers at gage. It also has 8 nozzles that can go to 22/32". It has 2 1313 tip grind cutters per blade. The tool joint is 4-1/2" API (5" threads connecting the head and tool joint).

Features:

- Bullets set to engage at .077 in DOC. (156 rpm @ 60 ft/hr)

Bit Body:

Ø9-7/8”

5 Blades

3 primary blades

2 secondary blades

65/70/80/75/70 Blade angles

5° Cone

3” Gage (1" full/ 2" Tapered @ 1°)

Open Face Volume =  in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. |  |  |  |
| Center of round | 1.962 | 1.913 | 1.02 |
| Gauge | 1.796 | 1.809 | .993 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary.

Back rakes:

Cone 		20°

Nose 		20°

Shoulder 	20°

Gage		35°

Components:

08 - 22/32 Nozzle Sleeves

08 - 60 Series Nozzles

31 - 1613 Cutters

30 - 1313 Cutters

10 - 1/2" Diamond WP

6   - 1/2" PDC Knuckles

10 - 1/2" Carbide Knuckles