2173A Ø7-7/8” 616-SR+(K)

Center cutter and trimmers are 1613. Cutter position 1 through 14 are Saddle cutters. 2 backup cutters on primary blades only in XR configuration. Profile creates an active gage.

Features:

- Bullets and backups are set to engage at .3”/rev.

Bit Body:

Ø7-7/8”

6 Blades

3 primary blades

3 secondary blades

60/55/66/60/65/55 Blade angles

9° Cone

3” Gage

Open Face Volume = 20.779 in^3

Height to Width Ratio:

|  | Height (in) | Width (in) | Height/Width |
| --- | --- | --- | --- |
| 1/3 of Bit Rad. | 1.011” | 1.438” | 0.703 |
| Center of round | 1.256” | 1.782” | 0.705 |
| Gauge | 0.862” | 1.900” | 0.454 |

Primary Cutting Structure:

Non-redundant; Reverse Spiral; Secondary blades are flush to primary.

Back rakes:

Cone 		15°

Nose 		15°

Shoulder 	15-25°

Gage		30°

Components:

06 - 22/32 Nozzle Sleeves

06 - 60 Series Nozzles

32 - 1613 Cutters (14 Saddle cutters)

06 – 1313 Cutters

15 – ½” TCI Bullets